SMART BIN PROJECT

In-Video Tutorial Reference Guide

Jeff C137  |  AI Robotics Innovation Hub

# What Is This Project?

A Smart Bin is an automatic dustbin that opens its lid when you wave your hand near it. No touching, no button, no remote. It uses three core components: an IR sensor to detect your hand, an ESP32 microcontroller as the brain, and a 28BYJ-48 stepper motor to open and close the lid automatically.

Perfect for beginners! If you can plug in wires and upload code, you can build this.

# How It Works — 7 Simple Steps

| Step | What Happens | Component |
| --- | --- | --- |
| 1 | You wave your hand near the sensor | You! |
| 2 | Infrared beam is broken — signal sent to ESP32 | IR Sensor FC-51 |
| 3 | ESP32 reads LOW signal (Active LOW logic) | ESP32 Dev Board |
| 4 | Short beep confirms hand detected | Active Buzzer |
| 5 | Lid swings open 90 degrees | 28BYJ-48 Stepper Motor |
| 6 | Lid stays open for 5 seconds | Code: delay(5000) |
| 7 | Lid closes automatically | Stepper reverses 90 degrees |

# Recommended Parts

These are the exact components Jeff used in the final build. Purchase links in the YouTube description.

| Component | Recommended Model | Why We Chose It | Qty |
| --- | --- | --- | --- |
| Microcontroller | ESP32 Dev Board (38-pin) | More GPIO pins than Arduino Uno, built-in WiFi for future upgrades | 1 |
| IR Sensor | FC-51 Infrared Module | Short-range, reliable, not triggered by ambient movement like PIR | 1 |
| Stepper Motor | 28BYJ-48 (5V) | Precise 90-degree control, low cost, widely available worldwide | 1 |
| Motor Driver | ULN2003 Driver Board | Designed specifically for 28BYJ-48 — buy them as a kit together | 1 |
| Buzzer | Active Buzzer 5V | No extra code needed — just set pin HIGH to beep | 1 |
| Jumper Wires | M-F and M-M 20cm set | Essential for breadboard connections | 20+ |
| Breadboard | Half or full size | No soldering needed for prototyping | 1 |
| Power | 5V USB / Power bank | Keeps ESP32 and motor running cleanly | 1 |
| Dustbin | Any hinged-lid bin | Cardboard lid works great for your first prototype | 1 |

# Wiring Guide

Most build failures come from one wrong wire. Double-check every connection before powering on!

## IR Sensor (FC-51) to ESP32

| FC-51 Pin | ESP32 Pin | Note |
| --- | --- | --- |
| VCC | 3.3V | Sensor power |
| GND | GND | Ground — share with ESP32 |
| OUT | GPIO 33 | Active LOW — goes LOW when hand is detected |

## Active Buzzer to ESP32

| Buzzer Pin | ESP32 Pin | Note |
| --- | --- | --- |
| Positive (+) | GPIO 2 | Control signal from ESP32 |
| Negative (-) | GND | Ground |

## Stepper Motor (28BYJ-48 via ULN2003) to ESP32

| ULN2003 Pin | ESP32 Pin | Note |
| --- | --- | --- |
| IN1 | GPIO 14 | Coil sequence pin 1 |
| IN2 | GPIO 26 | Coil sequence pin 2 |
| IN3 | GPIO 27 | Coil sequence pin 3 |
| IN4 | GPIO 25 | Coil sequence pin 4 |
| VCC (+) | 5V | Motor needs 5V — NOT 3.3V |
| GND (-) | GND | Shared ground with ESP32 |

# The Code — Explained Simply

Upload this using Arduino IDE with ESP32 board support installed.

## Include Library and Define Constants

#include <Stepper.h>

      #define STEPS 2048           // 28BYJ-48 = 2048 steps per full rotation

      #define NINETY_DEG (STEPS/4) // 512 steps = exactly 90 degrees

2048 steps = one full 360-degree rotation. Dividing by 4 gives exactly 90 degrees for the lid.

## Define Pins and Create Motor Object

const int irPin     = 33;  // IR sensor OUT wire

      const int buzzerPin = 2;   // Buzzer positive wire

      Stepper myStepper(STEPS, 14, 26, 27, 25); // ULN2003 IN1-IN4

      bool isRunning = false;    // Prevents double-triggering

## Setup — Runs Once on Power-On

void setup() {

        pinMode(irPin, INPUT);

        pinMode(buzzerPin, OUTPUT);

        myStepper.setSpeed(15);     // 15 RPM — smooth and controlled

        Serial.begin(115200);       // For Serial Monitor debugging

        myStepper.step(-NINETY_DEG); // HOME: force lid CLOSED on startup

      }

The homing step ensures the lid always starts closed, even if it was left open from the last session.

## Main Loop — Runs Forever

void loop() {

        int irState = digitalRead(irPin);

        if (irState == LOW && !isRunning) { // LOW = hand detected (Active LOW!)

          isRunning = true;

          digitalWrite(buzzerPin, HIGH); delay(500); // Beep!

          digitalWrite(buzzerPin, LOW);

          myStepper.step(NINETY_DEG);    // Open lid

          delay(5000);                   // Wait 5 seconds

          myStepper.step(-NINETY_DEG);   // Close lid

          delay(200);

          isRunning = false;

        }

      }

Why LOW? The FC-51 uses Active LOW logic — it outputs LOW (0V) when it detects an object, HIGH when clear. This is the opposite of what you might expect!

# Why IR Sensor? (The Fail Story)

|  | PIR Motion Sensor (Version 1) | IR Sensor FC-51 (Final Design) |
| --- | --- | --- |
| How it detects | Detects heat from any movement in range | Detects object breaking infrared beam |
| Detection range | Up to 7 metres | 2 to 30 cm (adjustable with potentiometer) |
| False triggers | Very high — fans, people walking by, phones | Very low — only close objects in front |
| Stress test result | FAILED — opened every 30 seconds | PASSED — consistent and reliable |
| Jeff's verdict | Too sensitive for a dustbin | Perfect. Exactly what we needed. |

# Troubleshooting

| Problem | Likely Cause | Fix |
| --- | --- | --- |
| Lid won't open at all | IR sensor wiring wrong | Check OUT pin is on GPIO 33 and LOW logic is correct |
| Opens non-stop | IR sensitivity too high | Turn blue potentiometer on FC-51 anticlockwise |
| Never detects hand | IR sensitivity too low | Turn potentiometer clockwise slowly and test |
| Motor hums but doesn't move | Motor getting 3.3V not 5V | Move VCC wire to 5V pin on ESP32 |
| Lid doesn't close fully | STEPS value wrong | Confirm 28BYJ-48 and STEPS = 2048 |
| Code won't upload | Wrong board selected | Tools > Board > ESP32 Dev Module |

Full code, circuit diagram and GitHub link are in the description!

Subscribe to Jeff C137 — new builds every week!